UNIVERSITY LABORATORY REPORT

Design, Analysis and Fabrication of a

3R Serial Robotic Manipulator Arm

Submitted by:

[Student Name]

[Registration No.]

Supervised by:

[Supervisor Name]

Department of Mechatronics Engineering

May 2026

1.  Key Characteristics

The manipulator under study is a three-revolute-joint (3R) serial robotic arm. Each joint is an actuated revolute joint making this a fully rotational, open-chain mechanism with three degrees of freedom. The principal characteristics are summarised in Table 1.

Table 1: Key Characteristics of the 3R Manipulator

| Parameter | Value / Description |
| --- | --- |
| Configuration | RRR — three revolute joints in series (serial chain) |
| Degrees of Freedom | 3 |
| Joint 1 — θ₁ | Rotation about vertical Z₀ axis (base rotation), range: 0° – 180° |
| Joint 2 — θ₂ | Rotation about horizontal Z₁ axis (shoulder), range: 0° – 180° |
| Joint 3 — θ₃ | Rotation about Z₂ axis (elbow), range: 0° – 180° |
| Base Height (d₁) | 90 mm — vertical offset from base frame to shoulder joint |
| Link 2 Length (l₂) | 210 mm |
| Link 3 Length (l₃) | 170 mm (includes end-effector body) |
| Maximum Reach | 380 mm from shoulder (l₂ + l₃, arm fully extended) |
| Minimum Reach | 40 mm (¦l₂ − l₃¦, arm fully folded) |
| End-Effector | Point at tip of Link 3 — no gripper |
| Actuators | MG995 Metal Gear Servo Motors × 3 (torque: 9.4 kg·cm @ 6 V) |
| Controller | ESP32 Microcontroller — PWM via ledc peripheral |
| Link Material | Acrylic (PMMA) sheet, 5 mm thick, ρ ≈ 1180 kg/m³ |
| Design Software | SolidWorks |
| Analysis Software | MATLAB |
| Firmware IDE | Arduino IDE |

2.  Design

2.1  Mechanical Structure

The 3R manipulator consists of a fixed acrylic base plate, three MG995 servo motors acting as revolute joints, two flat acrylic link bars, and a point end-effector at the tip of Link 3. The base servo (Joint 1) is mounted vertically and rotates the entire arm assembly about the Z₀ axis. Link 1 is the rigid vertical column of height d₁ = 90 mm connecting the base to Joint 2. Link 2 (l₂ = 210 mm) is a flat acrylic bar connecting Joint 2 to Joint 3. Link 3 (l₃ = 170 mm) extends from Joint 3 to the end-effector point. A gripper-like profile visible at the end of Link 3 in the SolidWorks model is a structural feature of Link 3 only and carries no functional gripper purpose. All links are machined from 5 mm thick acrylic sheet.

[ INSERT IMAGE HERE ]

Figure 1: SolidWorks Assembly — 3R Manipulator (Isometric View)

2.2  Frame Assignment

Coordinate frames are assigned following the standard Denavit-Hartenberg convention. The Z-axis of each frame coincides with the joint rotation axis. The symbol ⊗ in the design sketches denotes a Z-axis directed out of the plane, confirming that Z₁ and Z₂ are horizontal and parallel. The base frame Z₀ points vertically upward.

[ INSERT IMAGE HERE ]

Figure 2: DH Frame Assignment Sketch (Side View with Z-axes and Origins labelled)

2.3  Home / Reference Configuration

Before executing any motion task the manipulator is driven to a fixed reference pose known as the home configuration. This ensures a consistent and safe starting state from which all trajectories are planned. The home joint angles are:

θ1 = 0°,   θ2 = 0°,   θ3 = 90°

Substituting into the forward kinematics equations (derived in Section 4):

x = cos 0° · (210 cos 0° + 170 cos 90°) = 1 · (210 + 0) = 210 mm

y = sin 0° · (210 cos 0° + 170 cos 90°) = 0 mm

z = 90 + 210 sin 0° + 170 sin 90° = 90 + 0 + 170 = 260 mm

The end-effector point is therefore located at (210, 0, 260) mm in the base frame at the home position. Table 2 gives the corresponding PWM initialisation values for the ESP32.

Table 2: PWM Values at Home Configuration

| Joint | Home Angle | PWM Pulse Width |
| --- | --- | --- |
| Joint 1 (θ₁) | 0° | 500 μs |
| Joint 2 (θ₂) | 0° | 500 μs |
| Joint 3 (θ₃) | 90° | 1500 μs |

The PWM frame frequency is 50 Hz. Pulse widths range from 500 μs (0°) to 2500 μs (180°) across all three servos. On power-up the ESP32 drives all channels to home values before accepting any motion commands.

[ INSERT IMAGE HERE ]

Figure 3: SolidWorks — Home Configuration (θ₁=0°, θ₂=0°, θ₃=90°) Side View

3.  Homogeneous Transformation Matrices

A homogeneous transformation matrix T ∈ ℝ⁴ˣ⁴ encodes the rotation and translation from one coordinate frame to the adjacent frame. Using the standard Denavit-Hartenberg convention, the transformation between frames i−1 and i is:

Ti−1i = Rot(Z, θi) · Trans(Z, di) · Trans(X, ai) · Rot(X, αi)

Expanding into matrix form, where cθ = cosθ, sθ = sinθ, cα = cosα, sα = sinα:

T = cθᵢ−sθᵢcαᵢsθᵢsαᵢaᵢcθᵢsθᵢcθᵢcαᵢ−cθᵢsαᵢaᵢsθᵢ0sαᵢcαᵢdᵢ0001

The shorthand notation cᵢ = cosθᵢ, sᵢ = sinθᵢ, c₂₃ = cos(θ₂+θ₃), s₂₃ = sin(θ₂+θ₃) is used throughout.

3.1  Individual Transformation Matrix T₀¹  (Joint 1)

Parameters: a₁ = 0, α₁ = 90° ⟹ cosα₁ = 0, sinα₁ = 1, d₁ = 90 mm, θ₁ variable.

⁰T₁ = c₁0s₁0s₁0−c₁0010900001

3.2  Individual Transformation Matrix T₁²  (Joint 2)

Parameters: a₂ = 210 mm, α₂ = 0° ⟹ cosα₂ = 1, sinα₂ = 0, d₂ = 0, θ₂ variable.

¹T² = c₂−s₂0210c₂s₂c₂0210s₂00100001

3.3  Individual Transformation Matrix T₂³  (Joint 3)

Parameters: a₃ = 170 mm, α₃ = 0°, d₃ = 0, θ₃ variable.

²T³ = c₃−s₃0170c₃s₃c₃0170s₃00100001

3.4  Cumulative Matrix  ⁰T²  =  ⁰T¹ · ¹T²

Multiplying row by column and applying c₁c₂ − s₁s₂ = c₂₃ where appropriate:

Element-by-element computation (row 1, column 1):

R(1,1) = c₁·c₂ + 0·s₂ + s₁·0 = c₁c₂

Element (1,4) — position x:

p_x = c₁·210c₂ + 0·210s₂ + s₁·0 + 0 = 210c₁c₂

Element (3,4) — position z:

p_z = 0·210c₂ + 1·210s₂ + 0·0 + 90 = 210s₂ + 90

Full result:

⁰T² = c₁c₂−c₁s₂s₁210c₁c₂s₁c₂−s₁s₂−c₁210s₁c₂s₂c₂0210s₂+900001

3.5  Full End-Effector Transform  ⁰T³  =  ⁰T² · ²T³

Multiplying ⁰T² by ²T³. Using the angle-sum identities:

c₂c₃ − s₂s₃ = cos(θ₂+θ₃) = c₂₃

s₂c₃ + c₂s₃ = sin(θ₂+θ₃) = s₂₃

Key element computations:

R(1,1) = c₁c₂·c₃ + (−c₁s₂)·s₃ = c₁(c₂c₃−s₂s₃) = c₁c₂₃

R(1,2) = c₁c₂·(−s₃) + (−c₁s₂)·c₃ = −c₁(c₂s₃+s₂c₃) = −c₁s₂₃

p_x   = 170c₁(c₂c₃−s₂s₃) + 210c₁c₂ = c₁(210c₂ + 170c₂₃)

p_z   = 170(s₂c₃+c₂s₃) + 210s₂ + 90 = 90 + 210s₂ + 170s₂₃

Final result:

⁰T³ = c₁c₂₃−c₁s₂₃s₁c₁(210c₂+170c₂₃)s₁c₂₃−s₁s₂₃−c₁s₁(210c₂+170c₂₃)s₂₃c₂₃090+210s₂+170s₂₃0001

4.  Forward Position Kinematics

Forward kinematics maps a set of joint angles (θ₁, θ₂, θ₃) to the Cartesian position (x, y, z) of the end-effector point O₃ in the base frame. The position is extracted from the fourth column of ⁰T³.

4.1  DH Parameter Table

Table 3: Denavit-Hartenberg Parameters

| Joint i | aᵢ (mm) | αᵢ (deg) | dᵢ (mm) | θᵢ (variable) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 90° | d₁ = 90 | θ₁* (0° – 180°) |
| 2 | l₂ = 210 | 0° | 0 | θ₂* (0° – 180°) |
| 3 | l₃ = 170 | 0° | 0 | θ₃* (0° – 180°) |

The asterisk (*) marks the variable (active) joint parameter. All remaining entries in each row are constant geometric properties of the link.

4.2  Forward Kinematics Equations

Extracting the position column of ⁰T³, the end-effector position O₃ is:

x  =  c1 · (210c2 + 170c23)

y  =  s1 · (210c2 + 170c23)

z  =  90 + 210s2 + 170s23

The x and y coordinates share the same horizontal radius r = 210c₂ + 170c₂₃, scaled by cosθ₁ and sinθ₁ respectively. This confirms that joint 1 sweeps the arm through the horizontal plane while joints 2 and 3 control vertical elevation.

4.3  Verification at Home Configuration

At θ₁ = 0°, θ₂ = 0°, θ₃ = 90° (home position):

x = cos0°·(210cos0° + 170cos90°) = 1·(210 + 0) = 210 mm

y = sin0°·(210cos0° + 170cos90°) = 0 mm

z = 90 + 210sin0° + 170sin90° = 90 + 0 + 170 = 260 mm

Result: O₃ = (210, 0, 260) mm — consistent with the home configuration derived in Section 2.3.

5.  Inverse Position Kinematics

Inverse kinematics (IK) computes the joint angles (θ₁, θ₂, θ₃) given a desired end-effector position (x_d, y_d, z_d). Two complementary approaches are presented below.

Define the following intermediate variables used by both approaches:

r  =  x2₝ + y2₞       [horizontal distance from Z₀ axis]

s  =  zd − d1  =  zd − 90       [height above Joint 2]

5.1  Algebraic Approach

Step 1 — Solve for θ₁

From the FK equations x = c₁·r and y = s₁·r. Dividing y by x:

tanθ1  =  ydxd

θ1  =  atan2 (yd,  xd)

The four-quadrant atan2 function is used to correctly handle all quadrants. The joint range 0°–180° restricts valid solutions to y_d ≥ 0.

Step 2 — Solve for θ₃

Squaring and adding the planar FK equations for r and s:

r  =  210c2 + 170c23

s  =  210s2 + 170s23

r2 + s2  =  (210c2+170c23)2  +  (210s2+170s23)2

Expanding and collecting using c₂c₂₃ + s₂s₂₃ = cos(θ₃):

= 210² + 170² + 2·210·170·cosθ₃

= 44100 + 28900 + 71400 cosθ₃

= 73000 + 71400 cosθ₃

Therefore:

cosθ3  =  r2 + s2 − 7300071400

θ3  =  ± arccosr2 + s2 − 7300071400

Two solutions exist: θ₃ > 0 (elbow-up) and θ₃ < 0 (elbow-down). A real solution exists only when |cosθ₃| ≤ 1, i.e. the target lies within the reachable workspace.

Step 3 — Solve for θ₂ (algebraic substitution)

Expanding the FK equations for r and s with θ₃ now known:

r  =  (210 + 170c₃)c₂ − 170s₃·s₂  =  k₁c₂ − k₂s₂

s  =  (210 + 170c₃)s₂ + 170s₃·c₂  =  k₁s₂ + k₂c₂

where the substitution constants are:

k1  =  210 + 170c3

k2  =  170s3

Solving simultaneously:

θ2  =  atan2(k1s − k2r ,  k1r + k2s)

5.2  Geometric Approach

Step 1 — θ₁ (identical to algebraic)

θ1  =  atan2(yd,  xd)

Step 2 — Distance D and θ₃ via Law of Cosines

In the vertical (r, s) plane, D is the distance from Joint 2 to the end-effector:

D  =  r2 + s2

Applying the law of cosines to the triangle with sides l₂, l₃ and hypotenuse D:

D² = l₂² + l₃² − 2l₂l₃cos(180°−θ₃) = 44100 + 28900 + 71400cosθ₃

cosθ3  =  D2 − 7300071400

Step 3 — Angles β and γ for θ₂

β is the elevation angle of the line from Joint 2 to the end-effector:

β  =  atan2(s ,  r)

γ is the interior angle at Joint 2, from the law of cosines:

cosγ  =  l2₂ + D2 − l2₃2l2D  =  D2 + 15200420D

γ  =  arccosD2 + 15200420D

The shoulder angle θ₂ is then:

θ2  =  β − γ     [elbow-up solution]

θ2  =  β + γ     [elbow-down solution]

6.  Velocity Kinematics

Velocity kinematics relates joint angular velocities to the linear and angular velocity of the end-effector through the Jacobian matrix J. The explicit geometric Jacobian method is used, following the instructor’s step-by-step procedure.

ṻₑωₑ  =  J · θ̇₁θ̇₂θ̇₃

6.1  Step 1: Extract Z-axis Vectors from Cumulative T Matrices

The Z-axis vector Zᵢ is the third column of the rotation sub-matrix of ⁰Tᵢ. From the matrices derived in Section 3:

Z0  =  001

(base frame Z-axis, pointing vertically upward)

Z1  =  Third column of R in ⁰T¹  =  s1−c10

Z2  =  Third column of R in ⁰T²  =  s1−c10

Note: Z₁ = Z₂ because α₂ = 0° introduces no twist between frames 1 and 2.

6.2  Step 2: Extract Origin Vectors from Cumulative T Matrices

The origin Oᵢ is the fourth column (position vector) of ⁰Tᵢ:

O0  =  000

O1  =  0090

O2  =  210c1c2210s1c290 + 210s2

End-effector origin (from ⁰T³):

O3  =  c1(210c2+170c23)s1(210c2+170c23)90+210s2+170s23

6.3  Step 3: Skew-Symmetric Matrices  S(Z)

The instructor requires that the cross product Z × v be written as S(Z)·v, where the skew-symmetric matrix of Z = [z_x, z_y, z_z]ᵀ is:

S(Z)  =  0−z_zz_yz_z0−z_x−z_yz_x0

S(Z₀) — for Joint 1

Z₀ = [0, 0, 1]ᵀ ⟹ z_x=0, z_y=0, z_z=1

S(Z₀)  =  0− 10100000

S(Z₁) — for Joint 2

Z₁ = [s₁, −c₁, 0]ᵀ ⟹ z_x=s₁, z_y=−c₁, z_z=0

S(Z₁)  =  00−c₁00−s₁c₁s₁0

S(Z₂) — for Joint 3

Z₂ = [s₁, −c₁, 0]ᵀ (same as Z₁)

S(Z₂)  =  S(Z₁)  =  00−c₁00−s₁c₁s₁0

6.4  Step 4: Compute  O₃ − Oᵢ  (Full Subtraction, No Premature Cancellation)

Let P = 210c₂ + 170c₂₃  and  Q = 210s₂ + 170s₂₃ for conciseness.

O₃ − O₀

O3 − O0  =  c1P − 0s1P − 090+Q − 0

=  c1Ps1P90 + Q

O₃ − O₁

O3 − O1  =  c1P − 0s1P − 0(90+Q) − 90

=  c1Ps1PQ

O₃ − O₂

O3 − O2  =  c1(210c₂+170c₂₃) − 210c1c2s1(210c₂+170c₂₃) − 210s1c2(90+Q) − (90+210s₂)

Expanding and grouping identical terms:

Row 1: 210c₁c₂ + 170c₁c₂₃ − 210c₁c₂  =  (210c₁c₂ − 210c₁c₂) + 170c₁c₂₃  =  170c₁c₂₃

Row 2: 210s₁c₂ + 170s₁c₂₃ − 210s₁c₂  =  (210s₁c₂ − 210s₁c₂) + 170s₁c₂₃  =  170s₁c₂₃

Row 3: 90+210s₂+170s₂₃−90−210s₂  =  (90−90)+(210s₂−210s₂)+170s₂₃  =  170s₂₃

O3 − O2  =  170c1c23170s1c23170s23

6.5  Step 5: Compute Linear Jacobian Columns  Jᵥᵢ = S(Zᵢ₋₁)(O₃−Oᵢ₋₁)

Column 1: Jᵥ₁ = S(Z₀)(O₃−O₀)

Performing the matrix–vector multiplication row by row:

Row 1: (0)(c₁P) + (−1)(s₁P) + (0)(90+Q)  =  −s₁P

Row 2: (1)(c₁P) + (0)(s₁P) + (0)(90+Q)  =   c₁P

Row 3: (0)(c₁P) + (0)(s₁P) + (0)(90+Q)  =   0

Jv1  =  −s₁P  c₁P    0

Column 2: Jᵥ₂ = S(Z₁)(O₃−O₁)

Row 1: (0)(c₁P) + (0)(s₁P) + (−c₁)(Q)  =  −c₁Q

Row 2: (0)(c₁P) + (0)(s₁P) + (−s₁)(Q)  =  −s₁Q

Row 3: (c₁)(c₁P) + (s₁)(s₁P) + (0)(Q)  =  c2₁P + s2₁P  =  P

Jv2  =  −c₁Q−s₁Q    P

Column 3: Jᵥ₃ = S(Z₂)(O₃−O₂)

Row 1: (0)(170c₁c₂₃) + (0)(170s₁c₂₃) + (−c₁)(170s₂₃)  =  −170c₁s₂₃

Row 2: (0)(170c₁c₂₃) + (0)(170s₁c₂₃) + (−s₁)(170s₂₃)  =  −170s₁s₂₃

Row 3: (c₁)(170c₁c₂₃) + (s₁)(170s₁c₂₃) + (0)(170s₂₃)  =  170c₂₃(c2₁+s2₁)  =  170c₂₃

Jv3  =  −170c₁s₂₃−170s₁s₂₃  170c₂₃

6.6  Step 6: Angular Jacobian Columns  Jωᵢ = Zᵢ₋₁

For revolute joints the angular velocity contribution is simply the Z-axis of the previous frame:

Jω₁  =  Z₀  =  [0,   0,   1]ᵀ

Jω₂  =  Z₁  =  [s₁, −c₁, 0]ᵀ

Jω₃  =  Z₂  =  [s₁, −c₁, 0]ᵀ

6.7  Step 7: Full Geometric Jacobian  J  (6 × 3)

Assembling all columns (P = 210c₂+170c₂₃,  Q = 210s₂+170s₂₃):

J  =  −s₁P−c₁Q−170c₁s₂₃c₁P−s₁Q−170s₁s₂₃0P170c₂₃0s₁s₁0−c₁−c₁100

The upper three rows form the linear velocity Jacobian Jᵥ and the lower three rows form the angular velocity Jacobian Jω.

6.8  Analytical (Linear Velocity) Jacobian  Jᵥ  (3 × 3)

Jᵥ  =  −s₁P−c₁Q−170c₁s₂₃c₁P−s₁Q−170s₁s₂₃0P170c₂₃

6.9  Singularity Analysis

A singularity occurs when the Jacobian loses rank, i.e. det(Jᵥ) = 0. The arm cannot generate end-effector velocity in one or more directions at a singular configuration.

Determinant Calculation — Cofactor Expansion Along Row 3

det(Jᵥ) = 0·C₃₁ + P·C₃₂ + 170c₂₃·C₃₃

Computing cofactor C₃₂:

C₃₂ = (−1)⁵ · det[ [−s₁P, −170c₁s₂₃], [c₁P, −170s₁s₂₃] ]

= −1 · [ (−s₁P)(−170s₁s₂₃) − (−170c₁s₂₃)(c₁P) ]

= −1 · [ 170s2₁Ps₂₃ + 170c2₁Ps₂₃ ]

= −1 · 170Ps₂₃(s2₁+c2₁)  =  −170Ps₂₃

Computing cofactor C₃₃:

C₃₃ = (+1) · det[ [−s₁P, −c₁Q], [c₁P, −s₁Q] ]

= (−s₁P)(−s₁Q) − (−c₁Q)(c₁P)  =  s2₁PQ + c2₁PQ  =  PQ

Therefore:

det(Jᵥ) = P·(−170Ps₂₃) + 170c₂₃·PQ

= 170P(−Ps₂₃ + c₂₃Q)

= 170P [ c₂₃(210s₂+170s₂₃) − s₂₃(210c₂+170c₂₃) ]

= 170P · 210 [ c₂₃s₂ − s₂₃c₂ ]

= 170 · 210 · P · sin(θ₂ − (θ₂+θ₃))

= −35700 · P · sinθ₃

det(Jᵥ)  =  −35700 · (210c2 + 170c23) · sinθ3

Singular Configurations

The determinant vanishes under two independent conditions:

Singularity 1 — Elbow Singularity:

sinθ3  =  0    ⟹    θ3  =  0°   or   180°

At θ₃ = 0° the arm is fully extended; at θ₃ = 180° the arm is fully folded. Joints 2 and 3 become collinear and lose one degree of independent control.

Singularity 2 — Workspace Boundary (Axis Alignment):

P  =  0    ⟹    210c2 + 170c23  =  0

The end-effector lies on the vertical Z₀ axis (r = 0). The azimuthal angle θ₁ becomes undefined and joint 1 is singular.

7.  Dynamics

The equation of motion of a rigid-body serial manipulator, derived from the Euler-Lagrange formulation, is:

M(q)äq  +  C(q, ̇q) ̇q  +  G(q)  =  τ

where q = [θ₁, θ₂, θ₃]ᵀ is the joint vector, M(q) is the 3×3 mass matrix, C(q,̇q) is the Coriolis and centrifugal matrix, G(q) is the gravity vector, and τ is the joint torque vector.

Note: Physical link masses are to be measured experimentally and will be substituted into the symbolic expressions below. Placeholder symbols m₂, m₃ denote link masses and I₂, I₃ denote link moments of inertia.

7.1  Inertia Tensor of Each Link

Each acrylic link is modelled as a uniform rectangular bar of length l, width w, and thickness t. The inertia tensor about the link centre of mass (COM), with axes aligned with the principal body axes, is derived by direct integration:

Iᴄᴹ  =  m(w²+t²)/12000m(l²+t²)/12000m(l²+w²)/12

For thin acrylic sheet (t = 5 mm ≪ l, w), terms involving t² are negligible. The dominant inertia components are:

Link 2  (l₂ = 210 mm, mass m₂, width w₂):

I2yy  ≈  m2l2₂12  =  3675m2  mm²

I2zz  ≈  m2(l2₂+w2₂)12

Link 3  (l₃ = 170 mm, mass m₃, width w₃):

I3yy  ≈  m3l2₃12  =  2408.3m3  mm²

By the parallel-axis theorem (shifting from COM to proximal joint end):

I2,joint  =  m2l2₂12  +  m2l222  =  m2l2₂3  =  14700m2  mm²

I3,joint  =  m3l2₃3  =  9633.3m3  mm²

7.2  Mass Matrix  M(q)

Using the Lagrangian formulation T = ½̇qᵀ M(q) ̇q, the mass matrix is computed from the squared velocities of the link centres of mass. The total translational kinetic energy is:

T  =  ½ m2|vC2|2  +  ½ m3|vC3|2  +  rotational terms

After computing squared COM speeds and collecting terms by joint velocity products, the 3×3 symmetric mass matrix M(q) is:

M(q)  =  M₁₁000M₂₂M₂₃0M₃₂M₃₃

with elements:

M11  =  m2105c22  +  m3210c2+85c232

M22  =  m2·105²  +  m3(210² + 2·210·85·c3 + 85²)  +  I2,joint

M33  =  m3·85²  +  I3,joint  =  7225m3  +  I3,joint

M23  =  M32  =  m3(7225  +  17850c3)

M12  =  M13  =  0

The block-diagonal structure (M₁₂ = M₁₃ = 0) arises because joint 1 rotation about the vertical Z₀ axis does not couple inertially with joints 2 and 3 in the rotational component. The coupling M₂₃ depends on cosθ₃ and represents the inertial coupling between shoulder and elbow.

Table 4: Geometric Factors for Mass Matrix Elements

| Element | Expression | Notes |
| --- | --- | --- |
| M₁₁ | m₂(105c₂)² + m₃(210c₂+85c₂₃)² | Varies with θ₂, θ₃ |
| M₂₂ | 11025m₂ + (44100+35700c₃+7225)m₃ + I₂ | Varies with θ₃ |
| M₃₃ | 7225m₃ + I₃,joint | Configuration-independent |
| M₂₃ = M₃₂ | (7225 + 17850c₃)m₃ | Varies with θ₃, zero at θ₃=90° |
| M₁₂ = M₁₃ | 0 | No coupling with base rotation |

8.  Control

8.1  Basic Revision

PID Controller

A Proportional-Integral-Derivative (PID) controller produces a control output u(t) from the joint angle error e(t) = θ_d(t) − θ(t):

u(t)  =  Kpe(t)  +  Ki∫e(t)dt  +  Kddedt

In discrete form (implemented on the ESP32 at sampling period Tₛ):

u[k]  =  Kpe[k]  +  KiTsΣe[k]  +  Kde[k] − e[k−1)Ts

State-Space Representation

Single-joint dynamics in state-space form with state x = [θ, θ̇]ᵀ:

ẋ  =  Ax  +  Bu,     y  =  Cx

A  =  010−b/J,     B  =    01/J,     C  =  [1   0]

where J is the effective moment of inertia at the joint and b is the viscous friction coefficient.

8.2  Robot Control Strategies

Joint-Space Control

The simplest strategy treats each joint independently with a PID loop. The ESP32 maps target angle to PWM pulse width linearly:

Pulse width (μs)  =  500  +  θi180° × 2000

This approach ignores dynamic coupling between joints and is acceptable for slow, precise motions under light loading.

Computed Torque Method

The full dynamic model is inverted to linearise and decouple the system:

τ  =  M(q)(äq_d + Kdė  +  Kpe)  +  C(q,̇q)̇q  +  G(q)

This cancels the nonlinear dynamics, reducing the closed-loop system to three decoupled linear second-order systems. Accurate values of M, C, G (requiring measured link masses) are needed.

Task-Space Control

The Cartesian position error drives the control via the Jacobian inverse:

̇q  =  Jv⁻¹ẋ_d  +  Kp(x_d − xe)

The pseudo-inverse Jᵥ⁺ = Jᵥᵀ(JᵥJᵥᵀ)⁻¹ is used near singular configurations. This strategy is preferred for straight-line trajectory tracking in Cartesian space.

ESP32 Implementation Notes

- PWM frame frequency: 50 Hz on all three servo channels.

- Pulse width range: 500 μs (0°) to 2500 μs (180°).

- Control loop runs on Core 1; serial communication and trajectory interpolation run on Core 0.

- The ledc peripheral provides 14-bit PWM resolution (≈0.01° per step).

- On power-up, all joints are driven to home position before accepting commands.

- Software joint-limit enforcement prevents commands outside the 0°–180° range.

9.  Social and Economic Issues

9.1  Employment and Workforce Displacement

The increasing deployment of robotic manipulators in manufacturing and logistics raises concerns about workforce displacement. Repetitive assembly, welding, and pick-and-place tasks are most susceptible to automation. While robots reduce unit production cost and improve repeatability, they can displace semi-skilled workers in the short term. The economic consensus favours net long-term job creation through new industries such as robot manufacturing, programming, and maintenance, but the transition period requires retraining programmes and active policy support.

9.2  Safety

Robotic manipulators pose physical hazards including collision, pinch-point injuries, and unpredictable motion during faults. International standards (ISO 10218, ISO/TS 15066) mandate safety fencing, emergency stops, and force-torque limiting for collaborative robots. The 3R arm presented here operates at low torque (MG995 maximum 9.4 kg·cm) and low speed, placing it in the low-risk category. Safe operating zones, software joint-limit enforcement, and power-off-on-fault logic are implemented in the ESP32 firmware.

9.3  Economic Viability and Cost

A principal advantage of this design is its low cost relative to commercial 3-DOF arms. The bill of materials — three MG995 servos, acrylic sheet, ESP32 board, fasteners, and a power supply — totals approximately USD 15–25, compared with thousands of dollars for a commercial counterpart. This makes the platform accessible to university laboratories, STEM education programmes, and small-scale research groups in developing economies, democratising access to robotic manipulation technology.

9.4  Educational and Research Value

The project provides a tangible, hands-on platform for teaching kinematic theory (DH parameters, FK, IK), velocity analysis (Jacobian, singularities), and basic dynamics — concepts that are otherwise purely abstract in textbook form. Students engage directly with the gap between theoretical models and physical hardware constraints such as servo backlash, acrylic flex, and PWM resolution limitations.

9.5  Environmental Impact

Acrylic (PMMA) is a petrochemical-derived thermoplastic that is not readily biodegradable. Responsible disposal, recycling through specialist PMMA processors, or substitution with bio-based polymers should be considered at end-of-life. The low power consumption of the ESP32 and MG995 servos (total peak current below 3 A at 6 V, i.e. 18 W maximum) results in a minimal operational carbon footprint, especially when powered from renewable energy sources.

9.6  Intellectual Property and Open-Source Ethics

The design, firmware, and kinematic analysis developed in this project are intended for open academic dissemination. Sharing CAD files, MATLAB scripts, and Arduino code under an open-source licence (MIT or Creative Commons) promotes reproducibility and allows other researchers to build upon the work without intellectual property barriers — an important principle in publicly funded academic research.

References

[1]  J. J. Craig, Introduction to Robotics: Mechanics and Control, 3rd ed. Pearson Prentice Hall, 2004.

[2]  B. Siciliano, L. Sciavicco, L. Villani and G. Oriolo, Robotics: Modelling, Planning and Control. Springer, 2009.

[3]  M. W. Spong, S. Hutchinson and M. Vidyasagar, Robot Modeling and Control, 2nd ed. Wiley, 2020.

[4]  Espressif Systems, ESP32 Technical Reference Manual, v5.1, 2023. [Online]. Available: https://docs.espressif.com

[5]  Tower Pro, MG995 Metal Gear Servo Datasheet, 2020.

[6]  ISO 10218-1:2011, Robots and Robotic Devices — Safety Requirements for Industrial Robots. ISO, Geneva, 2011.

[7]  International Federation of Robotics, World Robotics Report 2023. IFR, Frankfurt, 2023.